1.0	Purpose

This procedure provides evidence on how the quality of product varies with time under the influence of varied environmental factors and enables to establish the shelf lives and recommended storage conditions.

2.0	Scope

This procedure primarily addressed the basic information required by health authorities in EC, Japan and USA.

3.0	Responsibility

The Quality Control Department is responsible to :

-	establish the stability program and select the batches to be introduced into the program

-	establish product stability specifications and execute stability studies;

-	ensure that stability samples are under the proper stability storage conditions and the required tests for each of the products/formulas are performed in a timely manner;

-	maintain and evaluate the obtained stability test data

4.0	Procedure

4.1 	Selection of Batches:

Stability information from accelerated and long-term testing is to be provided on three production batches of the same formulation and dosage form in the containers and closure proposed for marketing.

The overall quality of the batches of drug product placed on stability should be representative of both the quality of the raw materials used and the quality of materials produced on a manufacturing scale.

4.2	Test Procedures and Criteria:

Testing should cover those features susceptible to changes during storage and likely to influence quality, safety and/or efficacy. Analytical test procedures should be fully validated and the assays should be stability-indicating.

4.3	Specification:

Limits of acceptability should be derived from the respective product release specification. It will need to include specific upper limits for  degradation products.

4.4	Storage Test Conditions

Long-term testing: 		25  2° C  / 60  5% RH

Accelerated testing:		40  2° C  / 75  5% RH

The length of the studies and the storage conditions should be sufficient to cover storage, shipment, and subsequent use. The long-term testing will continue for a sufficient period of time to cover the proposed or current shelf life. Frequency of testing is every three months over the first year, every six months over the second year and then annually.

The accelerated testing will be continued for six months and testing frequency is on a monthly basis.

4.5	Packaging Materials

The testing should be carried out in the final packaging proposed for marketing.

4.6 	Documentation

The Quality Control Manager is responsible to maintain a bound log book for batches introduced into the stability program. The log book will contain the following information :

- Product

- Lot Number

- Packaging

- No. of packages

- Quantity per package

- Storage conditions

- Date of manufacture

- Date start

- Date end

4.7	Data Evaluation

4.7.1	The Quality Control Manager is also responsible to maintain a systematic approach in the presentation and evaluation of the accumulated stability data. Stability data file will be kept up-to-date and reviewed by the Quality Control Manager in a timely manner.

4.7.2	Non-satisfactory results are reported to the Plant Manager  for investigation.

5.0	References

pp. 1959 - 1963, General Information/Stability Testing <1196>, USP23