1.0	Purpose

This SOP outlines the sampling plan employed for packaging components.

2.0	Scope

This procedure applies to all incoming packaging components.

3.0	Responsibility

Sampler or the staff designated by the QC Manager for sampling shall clearly understand this SOP.

4.0	Procedure

4.1	Samples will be taken from the square root of the number of cases (X). X is rounded to a whole number.

4.2	Equal number of samples from each case is to be taken as per sample size shown below.

| MIL-STD-105E Sampling Plan for level II Normal |  |
| --- | --- |
| Number of Units per case | Sample Size |
| 1,201 - 3,200 | 50 |
| 3,201 - 10,000 | 80 |
| 10,001 - 35,000 | 125 |
| 35,001 - 150,000 | 200 |
| 150,001 - 500,000 | 315 |

4.3	Acceptance and rejection number will be determined by the Acceptable Quality Level (AQL) chosen, as defined in individual specification sheet of packaging components.