1.0	Purpose

To outline the procedures on handling and testing of raw material.

2.0	Scope

It applies to all incoming raw material.

3.0	Responsibility

3.1	QC Department is responsible for handling and testing of all incoming raw material.

3.2	The Quality Control Manager or designee has the authority to approve or reject raw material based on the QC analysis results and the established specification.

4.0	Procedure

4.1	Each analysis request will be logged into the Analysis Entry Logbook once the Good Received Note (GRN) is received from the Warehouse.

4.2	After sampling, all samples must be logged into the Sampling Logbook with the following information:

- Product name

- Product lot number

- Container number sampled

- Quantity sampled

- Sampling plan (SOP no.)

- Sampling date

- Initials of sampler

4.3	Testing

4.3.1	The raw material sample is assigned to a Quality Control Technician who will test the material for conformity to the specifications for purity, strength and quality indicated on the respective raw material specification.

4.3.2	Only identification test is required for testing provided that the Certificate of Analysis of the respective lot is approved by the Quality Control Manager.

4.3.3	Each raw material must be tested a minimum of once per year, in accordance with SOP No.9.23 “Re-evaluation of Raw Material”.

4.4	Release/Rejection

4.4.1	Release for Use

Any lots of component materials which meet the appropriate written specifications for purity, strength and quality are released for use by the Quality Control Manager or designee.  Such approval follows review of the results entered on the Quality Control Analysis Report and is indicated by signature, dating, and placing a “Check mark” next to the word “Released”.

4.4.2	Conditional Release

Raw materials may be conditionally released by the Quality Control Manager or designee. A conditional release permits the material to be used in manufacturing before all test results for “Release” are obtained. The finished product manufactured, using a raw material that is “Conditionally Released”, can not be “Released” by Quality Control for distribution until  that raw material has been formally “Released”. The minimum testing required for a conditional release is an Identification, and if required for manufacturing, an Assay. The word “Conditional” must appear on all Quality Control Analysis Reports effecting a conditional release.

4.4.3	Rejected Raw Material

Any lots or component material which do not meet the appropriate written specifications are rejected by the Quality Control Manager or designee. Such rejection is indicated by signature, dating, and placing a “check mark” next to the word “Rejected” on the Quality Control Analysis Report.

4.5	Upon completion of testing and release/rejection of a lot of raw material, the following information is logged into the Analysis Entry Logbook by the QC Manager or designee:

- Date of release

- Released by

- Analyst’s  raw data logbook no. and page