1.0	Purpose

This working instruction outlines the incoming raw material sampling procedures.

2.0	Scope

This procedure applies to all incoming raw materials intended for production use.

3.0	Responsibility

3.1	Quality Control Manager is responsible to ensure that samplers are properly trained and procedures are strictly followed.

3.2	Warehouse keeper and the QC sampler shall clearly understand and strictly follow this SOP.

4.0	Procedure

4.1	The Quality Control Manager shall initiate the necessary sampling once the Good Received Note (GRN) and the Certificate of Analysis (CoA) from the principle are received and the product has been placed under Quarantine.

4.2	Sampling will be carried out by designated Quality Control personnel.

4.3	Warehouse keeper must be notified before sampling proceeds. Sampling must be taken place with the presence of Warehouse keeper.

4.4	Sampling Plan

i.  Active and main cosmetic ingredients

Each container will be sampled.

ii.  Excipients

If not specified otherwise, the number of containers sampled is determined by the following formula. Number sampled (X) equals the square root of the quantity (N) plus 1 where N represents the number of units in each lot. X is rounded to a whole number.

4.5	The sample quantities taken from container shall be appropriate for tests to be performed and a second sample of equal size shall be taken and placed in a second containers as a reserved sample.

4.6	Sampling shall be conducted according to the following manner:

- clear the sampling area;

- wear gloves and mask;

- wipe up container before opening;

- weigh accurately to the nearest tenth of a gram and record;

- clean up all the sampling tools before proceeds to different lot

4.7	The collected samples must be properly labeled and the container which has been opened for sampling shall be marked ‘Sampled” to allow traceability. After sampling process, the sampling hole(s) should be closed with care to avoid cross contamination during storage.

4.8	The number of samples taken and the container number being sampled must be  recorded into the Sampling Logbook.

4.9	Sampler will initial the Sampling Logbook and shall include:

- material name;

- supplier name;

- inventory lot#;

- container # sampled;

- quantity of sample;

- sampling procedure #;

- number of samples taken;

- signature of the sampler;

- sampling date.

- Sampling guidance

  - Sampling tools such as knives, pliers, saws, hammers, wrenches, implements to remove dust (preferably a vacuum cleaner)

5.2.	Pictures of several sampling tools

  - Sampling guidance:

- Material to re-close the packages (such as sealing tape), as well as self-adhesive labels to indicate that a part of the contents has been removed from a package or container.

- Containers due to be sampled should be cleaned prior to sampling if necessary.

- The health and safety aspects should be taken care during

- sampling

- process

- .

- The container

used to store a sample should not interact with the sampled material nor allow contamination. It should also protect the sample from light, air, moisture, etc., as required by the storage directions for the material sampled.

- Microbiology sampling tools shall be sterilised prior to use

- Aseptic technique shall be used during sampling.